# Highway-Rail Grade Crossing Safety for School Bus Drivers
NHTSA School Bus Driver In-Service Safety Series

### Learning Objectives

At the completion of this module, participants will be able to:

- Describe the potential dangers at highway-rail grade crossings

- Explain the appropriate procedures to make such a crossing safely

### Time Required

1 hour

### Training Methods

- Participative lecture

- Guided discussion

### Training Materials

- PowerPoint slides and projection system

- Handouts: How to Cross a Highway-Rail Intersection Safely, Handout #1; Highway-Rail Grade Crossing Warning Devices, Handout #2; local policies and procedures

### Notes to Instructor

- Operation Lifesaver is a nationwide, nonprofit public information and education organization dedicated to eliminating collisions, injuries, and fatalities at highway rail-grade crossings.  For a comprehensive presentation of highway rail safety, call 800-537-6224 to arrange a certified Operation Lifesaver presenter.  For more information, go to www.oli.org.

| Lesson Plan | Instructor Notes |
| --- | --- |
| ## I. Introduction and Overview Say that you want to welcome everyone to the course. Introduce yourself. | 30 minutes Show Slide 1. |
| Explain that the purpose of the module is to review procedures at Highway-Rail Crossings, including how to read signs and interpret signals and how to respond to special circumstances. |  |
| - Explain that highway-rail grade crossing procedures apply everywhere – in every country, State, city, and county, and at every highway rail intersection. |  |
| - Stress the importance of the fact that, in a confrontation between a school bus and a train, the train ALWAYS wins. |  |
| Ask: How many of you have highway-rail intersections on your routes?. | Ask for a show of hands. |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ### Why Learn the Procedures? Explain that, whether or not you regularly encounter a highway-rail grade crossing, you need to know how to proceed safely at one. You never know when a detour or a new route will lead you to a railroad crossing. |  |
| Emphasize that, if you do cross a highway-rail grade crossing regularly, you should always expect a train. Freight trains don’t operate on set schedules and passenger trains change their schedules all the time. |  |
| ### Important Things to Know Ask: When you come up to a railroad crossing, who has the right-of-way?. Explain that the train does because a 100-car train traveling 55 mph can take a mile to stop – a distance equal to 18 football fields. | Answer: Always the train. |
| Ask: How far away from the tracks should you stop?. | Answer: At least 15 feet but no more than 50 feet. |

| Lesson Plan | Instructor Notes |
| --- | --- |
| Ask: What does this sign mean?. | Show Slide 2. Answer: The crossbuck warns that there is a grade crossing just ahead. It means the same as a yield sign. |
| Explain that, in order to ensure the safety of the driver and the children onboard the bus, it is important to know the answers to all of these questions and others related to highway-rail crossing. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ## II. Statistics and Key Points Highlight the value of safety training by explaining that collisions have decreased steadily from about 12,000 in 1972 to less than 3,000 in 2006. | 20 minutes |
| Say that the number is still more than it should be and then present the following statistics: - There are approximately 268,000 highway-rail and pedestrian crossings in the U.S. each year. - Most highway traffic fatalities occur within 25 miles of the driver’s home. - In the U.S., a train collides with a vehicle or a person once every 175 minutes. - In an average year, more people die in highway-rail crossings than in commercial airline crashes. | Show Slide 3. |
| Reinforce these key points: - Always expect a train - Judging a train’s speed and distances is very difficult - Never try to beat the train - Obey the crossing signals - Watch for multiple sets of tracks | Show Slide 4. |

| Lesson Plan | Instructor Notes |
| --- | --- |
|  | Optical illusions make it difficult to judge the speed and distance of an approaching train. Relate this phenomenon to watching a plane come in for landing. It usually looks like it’s hanging in the sky, but we know that it’s actually approaching rapidly. The plane’s size and the angle of its approach fool our eyes. The same is true of approaching trains. Viewed from a crossing, railroad tracks produce the illusion of great distance. That’s because the parallel lines of the rail converge toward the horizon. (The same illusion is used in art classes to create perspective). The apparent convergence of the rail gives us the impression that the train is further from the crossing that it really is. In addition, the apparent size of the approaching train changes very slowly. This leads drivers to assume it is not only far away but moving slowly – a deceptive and dangerous combination of illusions. |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ### Procedure for Crossing Explain that you will carefully review the proper procedure for crossing a highway-rail intersection. A detailed description of the procedure will be followed by a simple five-step checklist to remember when you’re at a crossing. - When approaching a crossing, tap your brakes to test that they’re working and to alert motorists behind you that you will be stopping the bus. - Stay on the right side of the roadway. Check for traffic around you before you start to move at a crossing. Use a pull-out lane if one is available and turn on your flashers if necessary. - Choose an escape route in case your brakes fail or there is a traffic tie-up in front of or behind you. - Stop no closer than 15 feet and no farther than 50 feet from the tracks. - Keep your foot on the brake so that you can’t move or be shoved into the path of a train. - Open the driver’s window and the service door. - Turn off radios and noisy equipment and leave them off until you have completed the crossing and are away from the track. Some school bus drivers start preparing when they first see the R/R sign. They begin to turn everything off and to open the window. In this way, it doesn’t take as long once they stop. | Distribute Handout #1, How to Cross a Highway-Rail Grade Crossing Safely. Review it with participants. |

| Lesson Plan | Instructor Notes |
| --- | --- |
| - Signal the students to be quiet using a signal that you use only at railroad crossings. - Look and listen. - Start crossing when you are sure that you don’t see or hear a train or a warning whistle. - Before moving, close the driver’s window and the service door. | Have the drivers show/describe their signals. Some school bus drivers turn the dome light on. Others use a hand signal, such as a V. |
| Introduce the Five Alive Safety Drill. This drill outlines the five easy steps to remember at a highway-rail grade crossing. | Continue to review Handout #1. |
| ### Step 1 Approach with care – Slow down as you near the railroad crossing and prepare to stop. |  |
| ### Step 2 Quiet – Alert students for quiet, turn off radio and fan, and listen for a train. |  |
| ### Step 3 Stop, look and listen – Stop no closer than 15 feet and no farther than 50 feet from the crossing. Open windows and doors, look both ways carefully and look and listen for a train. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ### Step 4 Double take – Look again in both directions. |  |
| ### Step 5 GO! – Do not switch gears while crossing. |  |
| ### What Not To Do At a Crossing Reinforce the things not to do at a railroad crossing: - Never stop on the tracks - Never stop within 15 feet of the tracks - Never try to back up once you’re on the tracks - Never change gears on the tracks - Never pass on the tracks | Review box on second page of Handout #1. Show Slide 5. |
| Ask: What should you do if the gate comes down after you have started across?. | Answer: Keep going, even if it means you will break the gate. |
| Ask: What should you do if the bus stalls or is trapped on the tracks?. | Answer: Get everyone out of the bus and off the tracks immediately. Explain that you will discuss the procedures for evacuating the bus on the tracks in more detail later. |

| Lesson Plan | Instructor Notes |
| --- | --- |
| Encourage drivers to know the size of their bus. - Make sure there is enough room for your bus on the other side of the tracks before crossing. - If there is any doubt whether you will fit without hanging over the tracks, do not proceed. Remember, the train will be 3 feet wider than the rails on both sides. - For example: It would take a 40-foot long school bus at least 5 seconds to travel 75 feet and clear the hazard zone (assuming an average speed of 10 mph after starting from a full stop). |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ## III. Signs and Signals Explain that passive signs and pavement markings, and active traffic control devices are installed along the roads near railroad tracks to regulate, warn and guide traffic. | 10 minutes Distribute Handout #2, Signs. Have participants follow along as you review the information |
| ### Advance Warning Signs - Yellow circular advance warning signs: Placed ahead of a public highway-rail intersection. They remind the driver to slow down, look and listen for a train, and be prepared to stop if a train is coming. - Pavement markings: Placed on paved roads near the yellow circular advance warning sign. They mean the same as the advanced warning sign and alert drivers that the road crosses railroad tracks ahead. - There may be a white stop line painted on the pavement just before the railroad tracks. The front of the school bus must remain behind this line while stopped at the crossing. - Parallel track signs: Diamond-shaped with black illustration. They warn drivers who are making a turn that there is a highway-rail intersection immediately after the turn. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ### Crossbuck Signs - These are the most common signs at public highway-rail intersections. They require you to stop and yield the right of way to a train if one is approaching. A sign below the crossbuck sign might indicate the number of tracks present. - If there is no stop line painted on the pavement, you must stop the bus before the crossbuck sign. |  |
| ### Gates/Lights - The gates are down and the lights are flashing when a train is present. Not every rail crossing has both gates and lights. Never attempt to go around the gates. |  |
| ### Other Signs - STOP signs: A stop sign at a rail crossing means the same as it does at a highway intersection. Stop, look, and listen for the train before proceeding. - Multiple tracks signs: When there is more than one set of tracks at a crossing there may be a sign beneath the crossbuck with a number indicating how many tracks are present. Watch for additional trains coming from either direction. |  |
| - DO NOT STOP ON TRACKS signs: This sign may be posted on the right side of the road or on the far side of the tracks. When you stop, be sure that the front – or the rear – of the bus is at least 15 feet from the tracks. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| - EXEMPT signs: These are sometimes posted below the crossbuck signs or advance warning signs. They alert the driver that the crossing has been abandoned or its use discontinued. A stop is not required unless a train is approaching or occupying the crossing, or the driver’s view don the tracks is blocked. Follow your local procedures about stopping at an EXEMPT crossing. - TRACK OUT OF SERVICE signs: In some States, these signs may be posted at crossings that have been abandoned or discontinued. Follow your local procedures about stopping at a TRACK OUT OF SERVICE sign. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ## IV. Special Procedures/Warnings Explain that it’s important to know what to do if you think signals at a railway crossing are malfunctioning, if your view is obstructed, or in the event that you get stuck on the tracks. Drivers must also understand local laws and procedures with respect to crossings. | 10 minutes Review the laws (State and local) and local procedures that apply to your district and present them in the discussion. |
| ### What If There Are Malfunctioning Signals? - If you’re at a crossing where the lights continue to flash and no train appears, do not cross. Call your dispatcher or the police. An 800 number is posted at some crossings. - If a uniformed law enforcement office or a railroad flagman at the crossing is present, ask if this person knows if the track is closed to rail traffic before moving your bus. If this person says it is OK to cross, you do not need to call your dispatcher. |  |
| ### What If There’s An Obstructed View? - Plan your route so it provides an adequate sight distance down the tracks in both directions at highway-rail grade crossings. Do not attempt to cross the tracks unless you can be sure that no trains are approaching. |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ### How Can I Plan Ahead For An Emergency? If your bus stalls or is trapped on the tracks or you are required to evacuate your bus for any reason, follow these steps or those laid out by your school district: |  |
| - Ahead of time: - Scout the crossings on your route - Know the safest location to take your students in the event of a crisis at any crossing on your route. - Plan how you would evacuate your bus - During an evacuation: - Get the students out quickly, without panic, and away from the tracks - Once you are certain all students are safely evacuated, call your Dispatch |  |

| Lesson Plan | Instructor Notes |
| --- | --- |
| ## V. Summary Reemphasize the following main points: - You cannot be too careful in crossing a highway-rail intersection. - Do not be tempted to short-cut the procedures we have outlined. - Make sure that everyone on your bus gets over a highway-rail grade crossing safely. - Make sure that your students know the quiet signal for highway-rail grade crossing. - Be familiar with the local laws and procedures, and have an evacuation plan in place should you get stuck on the tracks. | 5 minutes Show Slide 6. |