PROGRAM: FIR100				PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: C. Gabler, J. Marcus

PROGRAM FUNCTION AND METHODS:

FIR100 performs 100 Hz finite impulse response filtering of a UDS file

containing a digitized time series.  FIR100 is general purpose in

nature, but is used primarily to filter side impacted thoracic and

pelvic signals.

The program filters a signal using a four step process.  First, FIR100

uses a four-pole, zero-phase Butterworth filter with a 300 Hz cutoff.

This is done to prevent aliasing in the next step, subsampling to a

1600 Hz sampling rate.  After the anti-aliasing filter, the signal is

subsampled to exactly 1600 Hz.  (time between data samples = .000625

seconds). This is the required sampling rate for FIR100 to filter to

the correct frequencies.  At the user's option, any bias in the signal

is next removed.  Finally, the finite impulse response filter is

applied to the signal, and the signal is output in UDS format.

The resulting passband frequency is 100 Hz, the cutoff frequency is 136

Hz, the stopband frequency is 189 Hz, the passband ripple is 0.0225 dB,

and the stopband gain is -50 dB.

FIR100 started out life as the BIOPROC, and is the functional

equivalent of BIOPROC.  However, FIR100 has been updated to include a

more standard user interface (e.g. default output file names), NHTSA

standardized filtering software subroutines, and numerous code

modifications to enhance portability.  The BIAS, IIRFIL, and FIRF16

subroutines were coded by M.R. Neale of VRTC.

PROGRAM FEATURES/OPTIONS:

o  FIR100 provides the user with option to remove bias from the signal.

SETUP/RUN INSTRUCTIONS:

An existing UDS time series file must be supplied to the program.