PROGRAM: Diffr				PROGRAM MODIFICATION DATE:	6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	6/95

AUTHOR: Stuart G. Mentzer

PROGRAM FUNCTION AND METHODS:

Diffr performs differentiation of UDS files using a centered

difference formula that is suitable for a wide variety of signals.

PROGRAM FEATURES/OPTIONS:

For y-series data Diffr uses a centered difference formula of the form:

y'(i) = ( y(i+1) - y(i-1) ) / ( 2 delta_x )

with any appropriate units conversion factors.

The tail points can't use a centered difference since data is only

available on one side.  For the start tail Diffr uses a weighted

average of the three point interpolation formula:

y'(i) = ( -3 y(i) + 4 y(i+1) - y(i+2) ) / ( 2 delta_x )

and the linear extrapolation of the centered difference formula:

y'(i) = 2 y'(i+1) - y'(i+2)

= ( -2 y(i) + y(i+1) + 2 y(i+2) - y(i+3) ) / ( 2 delta_x )

Taking 1/3 of the first formula plus 2/3 of the second gives the

formula:

y'(i) = ( -7 y(i) + 6 y(i+1) + 3 y(i+2) - 2 y(i+3) ) / ( 6 delta_x )

The end tail is based on the analogous symmetric formula.  This

tail treatment works well for a wide variety of smooth and noisy

signals.

For x-y data Diffr uses the more general 3-point interpolation

formula (which simplifies to the centered difference formuala

for y-series).  The tail values are again based on the same

weighted average of the three point interpolation and extrapolated

centered difference formulas.  Special formulas are used for points

where the x values change direction or where very small x increments

would otherwise lead to serious roundoff errors.

By default Diffr uses a formula-computed initial (x=0) value for the

derivative signal.  The INI_VAL command line option tells Diffr to

use a file specification value, if appropriate, (such as INIVEL) for

the initial value.  Alternatively, a specific initial value can be

entered with the INI_VAL=<value> form of the option.

SETUP/RUN INSTRUCTIONS:

An existing UDS file must be supplied to the program.

KNOWN PROBLEMS/CONSTRAINTS:

Numerical differentiation is always a noise amplifying process,

and Diffr may produce noisy signals, especially if the data is

somewhat noisy to begin with and/or if a second differentiation

is performed.  The SimFil program may be a better tool for

directly computing smooth derivatives.