PROGRAM: RmBias				PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: Andrew Orndorff & Stuart G. Mentzer

PROGRAM FUNCTION AND METHODS:

RmBias performs bias removal on a signal using a user-specified

percentage of the signal to determine the size of the bias. Once

the portion of the signal to be used is specified RmBias uses an

averaging algorithm to determine the average magnitude of the

function over the interval. This average value is assumed to be the

bias, and the signal is then corrected by subtracting this bias

from each point on the signal.

PROGRAM FEATURES/OPTIONS:

RmBias provides the option of determining the bias from either the

start or end tails of the signal, as specified by the user.  Once the

user specifies which method to use, RmBias prompts for what percentage

of the signal to use in determining the bias.

RmBias can process either Y or X-Y channel format data.  In both cases

the bias is computed by averaging the y-axis data points over the

specified span.

SETUP/RUN INSTRUCTIONS:

Existing UDS files must be supplied to the program.

KNOWN PROBLEMS/CONSTRAINTS:

As with any other signal correction algorithm that allows user control

of the method, a reasonable sense of how much of the signal is useable

for bias determination is required. Although using the entire signal is

acceptable, this may not always produce the best results. Also, using

too small a portion of the signal may also generate improper results,

especially if the signal has not been filtered before bias

determiniation - excess noise may lead to an inaccurate bias value.

The use of tail averaging assures that the approximate first integral

is zero over the bias-corrected tail for y-series channel data.  This

does not hold for X-Y channel data.