PROGRAM: ATB2UDS			PROGRAM MODIFICATION DATE:	 6/95

SUPPORT: ISSI				DOC FILE MODIFICATION DATE:	 6/95

AUTHOR: Randa Radwan (ATB2UDS v2.0), Seth Grimes (v4.2)

PROGRAM FUNCTION AND METHODS:

The ATB2UDS program extracts motion and force histories from ATB

output (CVS version 4.2) and writes those to UDS files.

The program inputs the post-processing time history data file (FORTRAN

unit No. 8) which can be output by ATB. The generation of this file by ATB

is controlled by the value of NPRT(4) on ATB input card A.5. A postive value

of +1 to +4 will result in the above file being output.

Please refer to pp. 30-31 and 90-93  of the ATB (CVS) User's Manual

(VOLUME III) for more detail on the generation and use of the ATB output

post-processing time history data file.

PROGRAM FEATURES/OPTIONS:

ATB2UDS is basically a conversion program. It acquires the AVAILABLE

information, regarding the time histories, from the ATB output file and

writes it to UDS files.

ATB2UDS Options

---------------

The program provides UDS file output of the following time histories

as acquired from the ATB output file :

o  linear accelerations :  components and resultant of linear

acceleration (in local segment reference)

for points on segments - (as specified by

ATB input cards H.1)

o  linear velocities :     components and resultant of linear

velocities (in vehicle reference)

- (as specified by ATB input cards H.2)

o  linear displacements    components and resultant of linear

positions (in vehicle reference)for points

on segments - (as specified by ATB input

cards H.3)

o  angular accelerations   components and resultant of angular

accelerations (in local segment reference)

for segments - (as specified by ATB input

cards H.4)

o  angular velocities      components and resultant of angular

velocities (in vehicle reference) for

segments - (as specified by ATB input

cards H.5)

o  angular displacements   components and resultant of angular

rotations (in vehicle reference)

for segments - (as specified by ATB input

cards H.6)

0  plane forces            contact forces for the allowed contacts

between planes and segments - (as

specified by ATB input cards F.1)

---------------------------  NOTE --------------------------------------

The plane forces, for the allowed contacts, are ALWAYS available in the

ATB post-processing file; while the motion histories are optional and are

only available if requested on ATB input cards H.1 through H.6.

------------------------------------------------------------------------

ATB2UDS Features

----------------

1. HEADER INFORMATION

The following variables are set for each curve (history) in an output

UDS file :

- TSTPRF

- TSTSRC

- TSTREF (** This is set to the first 10 characters of the ATB

run description as entered on card A.2, to map the UDS files

with the specific ATB run)

- TSTNAM (** This is input by the user to qualify a given ATB run.)

- CURNAM

- CURTYP

- SENATT

- AXIS

- YTYP, YUNITS, XTYP, XUNITS

- NFP

- NLP

- DEL

- STATUS

- RD1, RD2, and RD3 (set to co-ordinates of the point on a given

segment for which linear motion histories are output)

- CURDSC

2. OUTPUT DATA SAMPLING RATE (DEL)

The time interval for data output, DT (as specified by the ATB input

card A.4), is not available from the ATB output file. ATB2DUS computes

the smallest time interval at which data is available and outputs the

corresponding number of points to the UDS files. Thus, it is possible to

get a finer sampling of the data then specified by DT as ATB can output the

data at the end of each successful integration step (See specification of

ATB input NPRT(26) card A.5).

To control the amount of printout from a regular ATB run but still

obtain a fine sampling of the histories, through UDS files from ATB2UDS,

you can specify a large value for DT relative to HMAX (on ATB input card

A.4) while specifying NPRT(4)=2 or 3 and NPRT(26)=0 (on ATB input card A.5).

3. UDS OUTPUT FILES NOMENCLATURE

The UDS output files are named Snnnnaa0b.mmm  such that:

nnnn :==   the value of TSTNAM as entered by the user

to qualify the given ATB run

aa   :==   AC, VC, DC, FC  corresponding to a computed

acceleration, velocity, displacement or

force history

b    :==   X , Y, Z and R for x, y, z components and

resultant of the motion histories  OR

P for the plane forces histories

mmm  :==   sequential curve number to uniquely qualify

each history (i.e. each output UDS file)

4. ATB2UDS OUTPUT LISTING FILE

The program generates a listing file, called ATB2UDS.OUT, which

contains the names of the output UDS files and a description of

their content.

SETUP/RUN INSTRUCTIONS:

The following inputs are required:

1. ATB output file name

2. An ATB run number, up to 4 digits (to be placed the UDS file TSTNAM

field and will be part of the UDS file names)

KNOWN PROBLEMS/CONSTRAINTS:

In addition to the above histories the ATB (unit 8) output file also

contains, if available from the simulated model, the following histories:

o  joint parameters :-

the lock condition, angles and torques for joints

(as specified by input cards H.7)

o  belt forces :- strain and anchor point forces for the allowed

contacts between belts and segments (as specified

by input cards F.2)

o  harness-belt forces :-

strain and endpoint forces for all belt sections

of the harness belts (from points specified as

endpoints on input cards F.8.d)

o  spring damper forces :-

(as specified by input cards D.8)

o  segment contact forces :-

contact forces for the allowed contacts between

segments and other segments (as specified by input

cards F.3)

o  airbag forces :-

airbag parameters and contact forces for the

allowed contacts between airbags and segments

(as specified by input cards F.6)

If the need arises, a future upgrade of ATB2UDS can be provided to write

these histories to UDS file format.

***********************************************************************

*******************************  NOTE *********************************

In future versions of ATB(CVS), there is a possibility that the

contents of the post-processing time history data file may be CHANGED!

Thus the ATB2UDS program MUST be changed accordingly!! The routines that

control/designate the output of the time histories in the ATB program

are OUTPUT and HEDING.

***********************************************************************

***********************************************************************