UDS File Naming Convention

--------------------------

This is a guideline for NHTSA UDS file naming.  These conventions are meant to

apply to raw and processed test data and generated (e.g., simulation) data.

The purpose of this guideline is to provide unique file names that are

indicative of the file contents.  Raw test data is uniquely named by the test

and curve numbers, so this guideline focuses on how to modify a UDS file name

after processing a single file and how to generate a UDS file name after

combining UDS files.

Only the most common data entities are shown.  In other cases, it is expected

that unique names should be available which maintain the basic naming

structure.  Alternative uses for some of the name substrings is reasonable for

special applications.  Newly identified entities which are frequently used will

be added to this list.

The philosophy of UDS file naming is that programs should avoid requiring input

files to have particular types of names, but that adhering to this structure

will simplify the user interface to UDS file data.  Where reasonable, programs

should generate default file names that follow this convention.

--------------------------------------------------------------------------------

Columns:		12345678.abc

Possible Characters:	V1845AA0.001

B____CCA.AVG

C    DDB.A__

S    EFC.DIF

_    FLD.D__

GME.PRD

INF.P__

NQG.RES

OxH.RAT

P_M.R__

S P.SUM

T X.SLT

V _.SL#

_  .SLa

.SL_

.S__

._#_

.__s

Columns		Interpretations

-------		----------------------------------------------------------------

1		V - Vehicle test data

B - Biomechanics test data

C - Component test data

S - Signal waveform generated data / Simulated/generated data

_ - other data source code

2 - 5		1845 - test number

____ - other test name code

6		A - Acceleration / Angular Acceleration

C - Curvature

D - Displacement / Angular Displacement / Dimensionless

E - Energy

F - Force

G - Gauge Pressure

I - Impulse / Angular Impulse

N - Noise

O - Other

P - Pressure

S - Strain / Static force

T - Time / Torque

V - Velocity / Angular Velocity / Voltage

_ - other Y-data type code

7		A - As Measured

C - Computed

D - DFT

F - Filtered / Channel Failed

L - Aligned

M - Meaningless / Modified

N - No Data

Q - Questionable Data

S - Subsampled / Scaled

T - Truncated

x - X-data type code for X-Y files

_ - other data status code

8		0 - unfiltered/as delivered data

A - 15 Hz cutoff data (filtered or DFT)

B - 30 Hz cutoff data (filtered or DFT)

C - 60 Hz cutoff data (filtered or DFT)

D - 100 Hz cutoff data (filtered or DFT)

E - 300 Hz cutoff data (filtered or DFT)

F - 500 Hz cutoff data (filtered or DFT)

G - 1000 Hz cutoff data (filtered or DFT)

H - Biomechanics standard filter process data

M - Magnitude (DFT)

P - Phase (DFT)

X - nonstandard filtered or DFT data

_ - other processing description code

a - c		001 - curve number

AVG - Average (general)

A__ - Average (specific)

DIF - Difference (general)

D__ - Difference (specific)

PRD - Product (general)

P__ - Product (specific)

RAT - Ratio (general)

R__ - Ratio (specific)

RES - Resultant (general)

R__ - Resultant (specific)

SUM - Sum (general)

SLT - Sum of Load cells, Total (all load cells)

SL# - Sum of Load cells in column # (0-9, 0 => column 10)

SLa - Sum of Load cells in row a (A-D)

SL_ - Sum of Load cells

S__ - Sum (specific)

_#_ - Vehicle/component number (non-occupant attachment) (1-2)

_#_ - Occupant location code (add 5 for vehicle/component 2)

(1-9 or O => Other)

__s - Sensor attachment code:

O - Occupant (general)

H - Head

C - Chest

P - Pelvis

:

V - Vehicle (general)

E - Engine

:

I - Impactor (general)

C - Component (general)

_ - other sensor attachment code

___ - other data description code

--------------------------------------------------------------------------------